Quarries National Joint Advisory Committee (QNJAC)

Plant Safety

Information Sheet 6

March 2012

Mobile Impact Crushers

Service Engineers Safety Inspections

This Information Sheet has been developed by the Quarries National Joint Advisory Committee (QNJAC) to help quarry operators, contractors, managers and others learn how to make health and safety improvements in the quarry industry. This guidance represents good practice, which may go further than the minimum you need to do to comply with the law.

Approved by the Quarries National Joint Advisory Committee (QNJAC) – Version 1: Date 28 March 2012

SERVICE ENGINEERS SAFETY INSPECTION – IMPACTORS

It is important that a machine operator is able to do an "informed" and "knowledgeable check" of the equipment that they are going to operate on an employer’s behalf. There are legal requirements under the Provision and Use of Work Equipment Regulations 1998 (PUWER 98), Regulation 6, and ‘Quarry Regulations 1999, Reg 12  for all safety aspects of the equipment to be inspected, and actions specified to remedy any defects found before they can create any risks in use. These checks are commonly known as ‘Pre Start check’s’ and are widely carried out and recorded within the industry.

However, in most cases this is where good practice stops. To help with this QNJAC have produced an “SERVICE ENGINEERS SAFETY INSPECTION" that can be used by a trained and competent inspector / fitters at dedicated service intervals.

Given to the inspector / fitter beforehand to ensure checks are completed at the time of the inspection, these documents should be stored along with all service records, Pre Start Checks and Defect Reports for the equipment. Records should also remain with the item of plant if ever sold or moved to a different site.

Once these checks have been carried out, they will enable the employer / quarry operator to have a more detailed inspection of the equipment documented which may not be carried out by the operator due to the lack of time/expertise in such areas.

This document has been developed by the Quarries National Joint Advisory Committee (QNJAC) to assist in making health and safety improvements in the quarring industry. This guidance may go further than the minimum you need to do to comply with the law. It is not intended to be fully comprehensive and you may need to tailor it to your specific requirements.

ENGINEERS SAFETY INSPECTION – IMPACTORS

| PLANT | ID No. | HRS | DATE | ENGINEER |
| --- | --- | --- | --- | --- |

| DESCRIPTION | SERVICEABLE? (Y/N) | DEFECT – REMARKS |
| --- | --- | --- |
| Guards – in place & effective? |  |  |
| Emergency stops / Pullwires – in place & effective? |  |  |
| Tracking controls – consider handset, umbilical cord & jack plug condition & function |  |  |
| Isolation points – effective & allows for multi-hasp? |  |  |
| Access & egress – consider steps/ladders, handrails, walkways & self-closing gates |  |  |
| Scrapers, ploughs & skirts – consider condition & tension |  |  |
| Structural inspection of framework & chassis – inspect for cracks, distortions, rusting, etc. |  |  |
| Tracks – tension & condition |  |  |
| Environmental – inspect spray bars, nozzles, hoods, covers, etc. |  |  |
| Hydraulic pipes and rams – inspect for leaks or signs of wear / damage |  |  |
| Fuel hoses - inspect for leaks or signs of wear / damage |  |  |
| Batteries, cables & terminals – check condition & connections |  |  |
| Warning signs – in place & visible for hazards present? (e.g. noise, dust, ejected materials, high pressure fluid injection, etc.) |  |  |
| Fire extinguisher – correct type present & within test date? |  |  |
| Start-up / Tracking audible warning - working correctly? |  |  |

Additional Comments: