EVENT DATE   2010 	EVENT LOCATION: UK Site

DESCRIPTION OF THE EVENT AND PHOTOGRAPH(S)

The silo on site was in the process of being dismantled for relocation elsewhere, on lifting down a section of shell plate, the lifting eyes pulled out of their fixing nuts and the plate fell 5 metres to the ground. No persons injured as area no entry to pedestrians during lift.

| Initial Notification Contractor notified Management, task stopped while Full investigation took place HSE informed of lifting failure ## Reason for failure Mismatch of equipment – Eyebolts purchased separate from nuts. Eye bolt sold as size M20 but actually ## M18 and Nut purchased M20 Corrective actions Double shackle now used for lift rather than lifting eyes, risk assessment and safe system of work and associated documents reviewed. Root cause analysis completed ## Preventative actions Use double shackle as preferred method | , . |
| --- | --- |
| Eye bolts and nuts should be purchased together Check to be completed on Eyebolt size which is stamped on neck of bolt and slack nuts should be checked for during rigging operation |  |
| Distribution …… |  |