Control of Noise

| Prepare | Location? Distractions? Talk Aids? |
| --- | --- |
| Reason | Noise-induced hearing loss is the most common occupational health hazard there is. |
| Why | There is no satisfactory treatment for noise-induced hearing loss. When you’re deaf, you stay deaf. |
| Outline | This talk will cover hazards, controlling noise and ear protection. |

# Hazards

- Some of the following things used on site can be harmful to your hearing: compressors, breakers, circular saws, concrete mixers, chainsaws, generators, vibrating rollers and excavators.

- Even if you are not using the noisy piece of equipment, you could be affected by someone using it close by.

- Look out for noise hazard signs on site and obey them.

- It’s not only on site that you have to remember to protect your hearing but after work also – noisy clubs, hi-fis, etc.

Q: Name some of the noise hazards to be found on site.

Controlling Noise

- Use a less noisy process if possible.

- If shouting is necessary in order to be heard, the noise level is high and you should wear ear protectors.

- Keep compressor covers closed when in use.

- Ensure breaker mufflers are correctly fitted.

Q: When should hear protection be worn?

- Don’t keep machinery running unnecessarily.

- Ensure you don’t expose workmates to your noise.

- Move noise source away from work area or move work area away from noise.

- If possible, shield noisy processes.  Work behind a wall or some other sound-absorbing material.

Q:  Name two precautions you can take with machinery to reduce noise.

# Ear Protection

- Don’t use cotton wool for ear protection, it is not effective

- Ensure earplugs are a good fit and correctly inserted.

- Regularly clean reusable earplugs.

- Use disposable earplugs once only.

- Clean your hands before touching all types of earplugs.

# Q: What should not be used instead of earplugs?

- Ear defenders should fit the head all round the seal.

- Ensure that ear defenders are worn the correct way round.

- Ensure defender seals are always in a serviceable condition.

- Don’t alter pressure of ear defenders by bending head band.

- If you have difficulty in wearing ear defenders, report it.

# Q:  What must you ensure when wearing ear defenders?

Do you have any questions for me?

Questions for you

Q: How can you reduce noise levels from a machine?

Q: What must you remember when handling earplugs?

REMEMBER  PROTECT YOUR EARING

Further information

- HSE publication – INDG127L Noise in construction