Safe Stacking of Materials

| Prepare | Location? Distractions? Talk Aids? |
| --- | --- |
| Reason | Unsafe stacking can lead to serious injuries. |
| Why | Don’t find yourself underneath an unsafe stack. |
| Outline | This talk will cover: general points and stacking of bricks, timber, pipes and prefabricated panels. |

# General points on Stacking

- When handling materials wear work gloves and boots as necessary, e.g. sharp edges or heavier loads.

- Only stack material in authorised areas. Never near doorways, accessways or on fire escape routes.

- Stack on a level surface and provide packing.

- Never make stacks higher than 3 times the minimum base width.

# Q: Where should materials never be stacked?

- Consider in what order materials will be unloaded from a stack and then load it accordingly.

- Stack close to work area to reduce amount of handling.

- If material is being lowered by machine, keep hands clear of load.

Q: What should you consider before loading material in a stack?

# Bricks, Blocks and Palleted Material

- Ensure base of stack is level.  Only stack two packs high.

- Ensure upper pack is loaded squarely on to lower one.

- If banding is damaged or materials are displaced in the pack, do not stack other materials on top.

- Leave sufficient space between pallets for safe removal.

# Q: if you see damaged banding, what should you not do?

# Timber

- Racks are the best method of storing small-sized timbers.

- Joists and larger timbers should be placed on bearers.

- Try to keep different lengths in separate stacks.

# Q: What should be used when stacking larger timbers?

Large Prefabricated Panels

- Stack flat or store secured in designed racks.

- Don’t lean against parts of semi-constructed buildings.

- Don’t lean against temporary structures.

- Don’t store upright where panels can be affected by wind.

# Q: Where and how should panels be stacked?

# Pipes and Tubes

- Where pipes are small in diameter, stack in racks.

- If large in diameter, securely chock at the base.

- Don’t stack in pyramids as they can become unstable.

- Large concrete rings must be laid flat so that they cannot be moved or rolled by any person, especially children.

# Q: How should you secure large diameter pipes?

Do you have any questions for me?

Questions for you

Q: How can you reduce the amount of handling?

Q: Where should you not stack large panels?

REMEMBER STACK SAFE – STAY SAFE