|  | Tool Box Talk ##### Working in Confined Spaces ##### Page 1 of 2 |  |
| --- | --- | --- |

What you need

Copy of Tool Box Talk

Copy of Permit To Work

Copy of FY/HS125

Copy of Local Information Sheets relating to Confined Spaces

Reason/Why?

People don’t always recognise what a confined space is and can under estimate the danger involved.

Learning Points

- The hazards

- Before entering a confined space

- Working in confined spaces

- What to do in an emergency

What are the hazards?

- Oxygen-depleted or enriched environments

- Presence of suffocating, toxic or flammable atmosphere

- Actual or potentially hostile environment (inside plant) becoming buried or drowned.

- Biological hazards, Weil’s disease from rat’s urine

- Confined Spaces include:	Cellars, chambers, pits, tanks, bunded areas, manholes, sewers, tunnels and some excavations

What should I do before entering a confined space?

- Don’t enter a confined space until a risk assessment has been carried out by a competent person. Assess if the work can be accomplished without entry first.

- If entry is required, ensure a Permit To Work has been completed first.

- Check for flammable or toxic gases and O2 content. Using gas monitoring device and check top, middle and bottom.

- If breathing apparatus is required, don’t enter the confined space until you are trained to use it.

- Ensure you have a second man outside of confined space for the entire time you are working.

- Make sure you can communicate with the second man throughout.

|  | Tool Box Talk ##### Working in Confined Spaces ##### Page 2 of 2 |  |
| --- | --- | --- |

Working in confined spaces?

- Work is controlled by a Permit to Work, which includes arrangements for rescue.

- Wear protective equipment and clothing as specified on Permit to Work.

- Only BASEFFA-approved electrical equipment is to be used where flammable gases may be present.

- Keep the gas monitor with you and always re-test the air after breaks.

- Don’t eat, drink, smoke or allow exhaust fumes in close proximity of point of entry.

What are the emergency procedures?

- Understand emergency procedure – know the location of first aid facilities and other emergency equipment.

- Make sure emergency / recovery apparatus that you are using is in working order.

- Don’t attempt a rescue without first raising the alarm and wearing breathing apparatus.

- The first duty of any rescuer is to ensure own safety.

Summary / Remember:

## DON'T ENTER THE SPACE UNTIL IT’S MADE SAFE!